CSE 332: Computer Organization and Architecture

ISA DESIGN

15/07/2018

Imran Hasan  Bhuiyan- 1612412042

Md. Rakib Imtiaz - 1610294642

Nayeem Mohammad – 1611553042

Sazzad Hossain - 1610139042

Introduction:

We are considering from the point of a computer architect who is hired by a vendor company. The idea is to design a new 12 bit single-cycle CPU with separate Data and Instruction Memory. We will decide the number and types of operand, number and types of instructions or operations, formats and list of registers.

Objectives:

To design an ISA that can solve high level language problem like simple arithmetic, logic operations, checking conditions and iterate through programs. It will also be able to accommodate external communication into ISA through input and output instructions.

Format: (Mixed)

R-type:

4 bit                                            4 bit                                             4 bit

| OP | Rs | Rt |
| --- | --- | --- |

I-type:

4 bit                                            4 bit                                             4 bit

| OP | Rs | Immediate |
| --- | --- | --- |

Operations:

Arithmetic:

1. add

2. addi

3. sub

4. mul

Logic:

1. and

2. or

3. sll

4. srl

Conditional/Unconditional (mixed) Branching operations:

1. beq

2. slt

3. jump (unconditional)

I/O and Data Transfer:

1. lw

2. sw

3. input

4. output

Operand:

Our design resemble MIPS instruction set in terms of operands. We will use 3 operands. Because it’s required to do simple calculations like addition (c = a + b). Requiring every instruction to have three fixed operands, no more and no less, conforms to the philosophy of keeping the hardware simple. Hardware for variable number of operands is more complicated than hardware for fixed number.

The type of our operands is register based. We opted for not using memory based      registers. Though the zero register will be a special kind of register which we will use

Table:

| Instruction | Type | Opcode |
| --- | --- | --- |
| add | Arithmetic | 0000 |
| addi | Arithmetic | 0001 |
| sub | Arithmetic | 0010 |
| mul | Arithmetic | 0011 |
| and | Logic | 0100 |
| or | Logic | 0101 |
| sll | Logic | 0110 |
| srl | Logic | 0111 |
| Beq | Branch conditional | 1000 |
| slt | Branch conditional | 1001 |
| jump | Branch unconditional | 1010 |
| lw | Data transfer | 1100 |
| sw | Data transfer | 1101 |
| Input | I/O | 1110 |
| Output | I/O | 1111 |

Register List :

We have selected registers from $zero, $t0-$t5, $s0-$s8 and assigned 4 bits for each of the register as our registers rs and rt contains 4 bits.

| Name of the Registers | Register Number | Value Assigned ( 4 Bits) |
| --- | --- | --- |
| $zero | 0 | 0000 |
| $t0 | 1 | 0001 |
| $t1 | 2 | 0010 |
| $t2 | 3 | 0011 |
| $t3 | 4 | 0100 |
| $t4 | 5 | 0101 |
| $t5 | 6 | 0110 |
| $s0 | 7 | 0111 |
| $s1 | 8 | 1000 |
| $s2 | 9 | 1001 |
| $s3 | 10 | 1010 |
| $s4 | 11 | 1011 |
| $s5 | 12 | 1100 |
| $s6 | 13 | 1101 |
| $s7 | 14 | 1110 |
| $s8 | 15 | 1111 |

add: It adds two registers and stores the result in destination register.

f = g + h

The variables f, g, h are assigned to the registers $s0, $t0, $t1

Operation: $s0 = $t0 + $t1

Syntax:  add $s0, $t0, $t1

sub: It subtracts two registers and stores the result in destination register.

f = g - h

The variables f, g, h are assigned to the registers $s0, $t0, $t1

Operation: $s0 = $t0 - $t1

Syntax: sub $s0, $t0, $t1

and: It AND’s two register values and stores the result in destination register. Basically, it sets some bits to 0.

f = g && h

The variables f, g, h are assigned to the registers $t1, $t2, $t3

Operation:  $t1= $t2 && $t3

Syntax: and $t1, $t2, $t3

or: It OR’s two register values and stores the result in destination register. Basically, it sets some bits to 1.

f = g || h

The variables f, g, h are assigned to the registers $t1, $t2, $t3

Operation:  $t1= $t2 || $t3

Syntax: and $t1, $t2, $t3

C code:  Calculate the sum of first 10 odd numbers

int i = 1, int n = 10, int sum = 0;

for(i=1; i<=10; i+=2) {

sum += i;

}

printf("The sum of all odd number from 1 to 10 is: %d", sum);

return 0;

Compiled MIPS code:

main:

addi $t1, $zero, 1

addi $t2, $zero, 9   # make $t2 9 to break the loop

addi $t3, $zero, 1

loop:

beq     $t3, 11, Exit   # check to see if $t3 = 11 if so exit

addi    $t3, $t3, 2 # change $t3 to next odd number by adding 2

add $t1, $t1, $t3   # add $t3 to $t1 (1+3,...3+5...etc...)

jump loop  #jump back to the start of the loop

Exit:

Addi $zero, $s0, 1   # system call code to print an int

lw $s1, num # address of int to print

syscall     # print the int

jump $ra  #exit

Format:

J – type:

4 bit                                                                                      8 bit

| Op | Jump |
| --- | --- |

Table:

| Instruction | Type | Opcode |
| --- | --- | --- |
| add | Arithmetic | 0000 |
| sub | Arithmetic | 0001 |
| addi | Arithmetic | 0010 |
| and | Logic | 0100 |
| or | Logic | 0101 |
| sll | Logic | 0110 |
| Beq | Branch conditional | 1000 |
| slt | Branch conditional | 1001 |
| jump | Branch unconditional | 1010 |
| lw | Data transfer | 1011 |
| sw | Data transfer | 1100 |
| In | I/O | 1101 |
| Out | I/O | 1110 |

Register List:

| Name of the Registers | Register Number | Value Assigned ( 4 Bits) |
| --- | --- | --- |
| $zero | 0 | 0000 |
| $t0 | 1 | 0001 |
| $t1 | 2 | 0010 |
| $t2 | 3 | 0011 |
| $t3 | 4 | 0100 |
| $t4 | 5 | 0101 |
| $s0 | 6 | 0110 |
| $s1 | 7 | 0111 |
| $s2 | 8 | 1000 |
| $s3 | 9 | 1001 |
| $s4 | 10 | 1010 |
| $in | 11 | 1011 |
| $out | 12 | 1100 |
| $eqval | 13 | 1101 |
| $base | 14 | 1110 |
| $one | 15 | 1111 |

Arithmetic:

Add:

add $s0 , $s1                 // s0 = s0 + s1;

Sub:

Sub $s0, $s1                  // s0 = s0 - s1;

Addi:

addi $s0, 5                    // s0 = s0 + 5;

Logical:

AND:

and $t0 , $t1                 // t0 = t0 & t1;

OR:

or $t0 , $t1                    // to = to | t1;

Shift left logical:

sll $s1, 2                       // s1 = s1 << 2;

Branch:

Be Equal:

beq $s1, L                // if ( s1 == eqval) then jump to L

Set less then:

slt $s1, $s2                  // s1 = (s1< s2) ? 1 : 0;

Jump:

jump l1;

Data Transfer:

Load word:

lw $s1, 3                                                   //  s1 = base + 4(3)

Store word:

sw $s1, 3                                                  //  base +4(3) = s1

Input:

In $in                                                                  // in = input

Output:

Out $out                                                               // display = out